Lesson 2: Reference Frames

MECH 637: Astrodynamic Reentry

Fall 2022

| Lt Col Robert A. Bettinger, Ph.D Office: Bldg 640, Room 345-F | E-mail: Robert.Bettinger@afit.edu Phone: 937-255-3636 x4578 |
| --- | --- |

Geocentric-Equatorial Coordinate Frame

- Frame notation: OXYZ

- Inertial reference frame with origin at planetary center

- x-axis ex: Points in direction of the Vernal Equinox

- z-axis ez: Points in direction of the North Pole

- y-axis ey: Orthogonal to the x-, z-axes; lies in the equatorial plane: ez=ex×ey

Planet-Fixed Coordinate Frame

- Frame notation: OX1Y1Z1

- Planet-fixed reference frame with origin at planetary center

- z1-axis ez1: Points in direction of the North Pole

- x1,y1 plane coincident with equatorial plane

- Assume planet rotates with a constant velocity: ω⨁=ω⨁ez1=ω⨁ez

- Coordinate transformation, where e and e1 represent the unit vectors of the OXYZ and OX1Y1Z1 frames, respectively:

e1=cosω⨁∆tsinω⨁∆t0-sinω⨁∆tcosω⨁∆t0001e
=Rzω⨁∆te

Where Rz represents the rotation matrix about the z-axis.

Vehicle-Pointing Coordinate Frame

- Frame notation: OX2Y2Z2

- Origin at planetary center

- x2-axis ex2: Points along vehicle’s position vector

- y2-axis ey2: Parallel to equatorial plane

- z2-axis ez2: Orthogonal to the x2-, y2-axes and completes right-handed system

- Coordinate transformation, where e1 and e2 represent the unit vectors of the OX1Y1Z1 and OX2Y2Z2 frames, respectively:

e2=cos-ϕ0-sin-ϕ010sin-ϕ0cos-ϕcosθsinθ0-sinθcosθ0001e1
=cosϕcosθcosϕsinθsinϕ-sinθcosθ0-sinϕcosθ-sinϕsinθcosϕe1=Ry2-ϕRz1θe1

Where θ is longitude and ϕ is latitude. Note: Coordinate transformation is accomplished by first multiplying e1 by Rz1θ, then the result by Ry2-ϕ.

- Coordinate transformation, where e and e2 represent the unit vectors of the OXYZ and OX2Y2Z2 frames, respectively:

e2=Ry2-ϕRz1θRzω⨁∆te

- Flight-Path Angle γ: The angle between the local horizontal frame (i.e., the plane passing through the vehicle and orthogonal to r) and the velocity v

  - γ  is positive when v is above the local horizontal plane

  - γ describes how much the velocity vector contributes to moving “in and out” along the radial vector

- Heading Angle ψ: The angle between the local parallel of latitude and the projection of v onto the local horizontal plane

  - ψ is positive in the right-handed direction about the x2-axis

  - ψ describes how much the velocity vector contributes to moving “toward and away” from the equatorial plane

Velocity-Referenced Coordinate Frame

- Frame notation: OX''Y''Z''

- Step 1: Create OX'Y'Z' by rotating the OX2Y2Z2 frame about the x2-axis by ψ so that the “new” y-axis, y', is aligned with the projection of v on the local horizontal plane:

ex'ey'ez'=ex2ey'ez'=1000cosψsinψ0-sinψcosψex2ey2ez2

e'=Rx2ψe2

- Step 2: Negatively rotate the OX'Y'Z' frame about the z'-axis by γ so that the “new” y-axis, y'', is aligned with v:

ex''ey''ez''=ex''ey''ez'=cos-γsin-γ0-sin-γcos-γ0001ex'ey'ez'
ex''ey''ez''=ex''ey''ez'=cosγ-sinγ0sinγcosγ0001ex'ey'ez'

e''=Rz'-γe'

- Final coordinate transformation:

ex''ey''ez''=cosγ-sinγcosψ-sinγsinψsinγcosγcosψcosγsinψ0-sinψcosψex2ey2ez2

e''=Rz'-γRx2ψe2

  - ey''-axis: Points along the velocity vector v; equivalently denoted as ev

  - ez''-axis: Perpendicular to both the velocity v and radial r vectors, and lies in the local horizontal plane

Relative Angular Motion

- Angular rotation is terms of the OX1Y1Z1 and OXYZ frames, respectively:

ω⨁=ω⨁ez1=ω⨁ez

Recall the transformation from the OX1Y1Z1 to the OX2Y2Z2 frame:

e2=cosϕcosθcosϕsinθsinϕ-sinθcosθ0-sinϕcosθ-sinϕsinθcosϕe1

ω1/0e2=cosϕcosθcosϕsinθsinϕ-sinθcosθ0-sinϕcosθ-sinϕsinθcosϕ00ω⨁

ω⨁=ω1/0e2=ω⨁sinϕex2+ω⨁cosϕez2

Note: ω1/0e2 indicates that the angular rate of OX1Y1Z1 relative to OXYZ is written in terms of coordinates along the e2 unit vectors.

- An equation for the overall angular rate can be assembled according to the transformation to e2 from e1 unit vectors:

Ω=θez1-ϕey2

When expressed entirely in the vehicle-pointing frame, the preceding equation becomes:

Ω2/1e2=Ω=θsinϕex2-ϕey2+θcosϕez2

Source Consulted

Hicks, Kerry D. Introduction to Astrodynamic Reentry, Second Edition. Dayton, OH: Hicks, 2014.